# 12. FLOW OVER SUBMERGED BODIES

| - When a fluid flows and passes the submerged body with a relative velocity (U∞), every differential surface area experiences a pair of forces due to Pressure & Wall shear stress(τ0). - Therefore, the body observes a net resultant force, which has 2 components, - Drag Force: Along the axis of U∞ - Lift Force: Perpendicular to the direction of U∞ |  |  |  |
| --- | --- | --- | --- |
| dFD=τ0dAcosθ+PdAsinθ | τ0dAcosθ= Shear or Friction Drag | PdAsinθ= Pressure Drag |  |

| CLASSIFICATION OF BODY |  |
| --- | --- |
| STREAMLINE BODY | BLUFF BODY |
| A body whose surface coincides with the streamlines of flow. | A body Which obstructs the streamlines of flow is known as bluff or blunt body. |
| Pressure Drag is very low & total drag is also less. | Relatively Higher Pressure Drag & Total Drag is high. |
| Boundary layer separation is low. Hence, lesser eddies and subsequent losses. | Boundary layer separation is high. Hence, Higher eddies and subsequent losses. |
| E.g. Fishes Shape, Aerofoil, Hydrofoils, etc… | E.g. Cylinders, etc… |

DRAG FORCE:

| FD=CDAch12ρU∞2 | CD=Coefficinet of Drag=f(Re, Geometry,Ma (For compressible flow)) Ach= Characteristic area |  |
| --- | --- | --- |
| Ach=AProj(For Blunt Body) |  | Ach=APlan(For Streamline Body) |

POWER LOST DUE TO DRAG: PD=FDU∞

FD mostly acting in the direction opposite to desired motion of the body.

LIFT FORCE:

| FL=CLAPlan12ρU∞2 | CD=Coefficinet of Lift=f(Re, Geometry,Ma (For compressible flow)) APlan= Plan area |
| --- | --- |

Note: If effective area is mentioned, then replace APlan&AProj With Aeff.

| DRAG ON SPHERES: - Pressure DragShear Drag=0.5 - If Re≤1, the flow is known as Stokes Flow. And Stokes Law is valid. | STOKES LAW: FD=3πμDU∞ For Stokes Law, CD=24Re ∵Ach=πD24 & Re≤1 |
| --- | --- |
| Q. Obtain Terminal Velocity of a sphere falling(down) under gravity in a dense and viscous fluid. Assume stokes law is valid. At equilibrium Condition, W=FB+FD⇒U∞=γb-γfD218μ | TERMINAL VELOCITY: The constant Velocity obtained by a body when it reaches equilibrium. |

DRAGS ON CYLINDERS:

| When the Re≥45, The vortices in the region of wake become highly unstable loading to shading of vortices. This phenomenon is referred as “Karman Vortex Trails” (Shown in fig.). |  |  |
| --- | --- | --- |
| Frequency of vortex Shading=n | Strouhal Number=nDU∞=0.1981-19.7Re |  |

If the Frequency of vortex shading matches with the natural frequency of cylinder, then resonance takes place. Due to resonance a sound is developed & it’s known as singing of wires.

MAGNUS EFFECT: The lifting effect generated on rotating body when it’s placed in a stream of flow. E.g. Lifting of table tennis ball When given a spin, etc…

| LIFT ON CYLINDER: FL=ρU∞ΓL=2πRρU∞VTL; ρ=Density of Fluid. U∞= Free Stream Velocity of Fluid relative to the body; Γ= Circulation =VT2πR. L= Length of the Cylinder. - CLon Cylinder Experiencing Magnus Effect =2πVTU∞ - Velocity on a cylinder Experiencing magnus Effect: VA=VT+2U∞sinθ Condition for a single stagnation point, VA=0 & θ=-90°⇒CL=4π |  |
| --- | --- |
|  | If VA<2U∞⇒CL<4π(2 St. Pt) |
|  | If VA=2U∞⇒CL=4π(1 St. Pt) |
| AEROFOILS: Stall Condition: α>α@maxLift Area =LC; Aspect Ratio =LC; L= Span; C= Chord. α= Angle of Attack. At stall Condition, 1) BL Separation take place, 2) CD drastically increases Circulation around an aerofoil, Γ=πC U∞sinα Lift Force on aerofoil, FL=ρU∞ΓL=πsinαCLρU∞2 For Aerofoil, CL=2πsinα |  |

| UNDER STEADY LIFT OF AEROPLANE: In the Steady State Condition, F =0 Forces Acting/Developed in the Aeroplane, FD= Drag Force; FL= Lift Force. W= Weight of The Aeroplane; FT= Thrust Developed by Aeroplane. Power Required to overcome Drag, PD=FDU∞ |  |
| --- | --- |